OpenShift Container Platform 4.20 on VMware vSphere
Complete Disconnected / Air-Gapped IPI Installation Runbook with HAProxy, Autoscaling and Auto-Healing

Version: 1.0 • Prepared: 26 August 2026
Target: OCP 4.20 • VMware vSphere • Disconnected/Restricted • IPI • HAProxy POC • Autoscaling • Auto-healing

# At-a-glance VM inventory

| Machine | Qty | OS | vCPU | RAM | Lifecycle / role |
| --- | --- | --- | --- | --- | --- |
| Connected mirror | 1 | RHEL 9.x | 4–8 | 16–32 GB | Persistent; Internet-side mirror host |
| Disconnected bastion | 1 | RHEL 9.x | 4 | 16 GB | Persistent; installer/admin |
| Mirror registry | 1 POC / HA prod | Supported registry | 4–8+ | 16–32+ GB | Persistent; must remain available |
| HAProxy | 1 POC / 2 prod | RHEL 9.x | 2–4 | 4–8 GB | Persistent; external LB |
| DNS | 2 recommended | DNS platform | 2 | 2–4 GB | Existing/redundant |
| NTP | 2 recommended | NTP platform | 2 | 2–4 GB | Existing/redundant |
| Bootstrap | 1 temporary | RHCOS | 4 | 16 GB | Created automatically; deleted after bootstrap |
| Control plane | 3 | RHCOS | 8 recommended | 32 GB recommended | Created automatically |
| Workers | 3 initial | RHCOS | 8 recommended | 32 GB recommended | Created automatically |
| Autoscaled workers | 0–N | RHCOS | per worker | per worker | Created automatically by Machine API |

# Document purpose

This runbook is a full implementation procedure for an OpenShift Container Platform 4.20 cluster on VMware vSphere using Installer-Provisioned Infrastructure (IPI) in a restricted/disconnected environment.

The design uses a user-managed external HAProxy load balancer for the proof of concept and deliberately keeps the OpenShift API and Ingress VIPs stable so the load-balancer implementation can later be replaced by NSX Advanced Load Balancer (Avi).

The document is written as a sequential runbook. Each phase identifies:
• which machine/operator performs the action;
• what must exist before the action;
• the exact command or configuration;
• what successful output/condition should look like;
• what to do before continuing.

Important: replace every example domain, IP address, password, username, vCenter path, datastore, cluster, network and registry value with values from your environment. Never copy example credentials into production.

# 1. Architecture and final target state

Steady-state OpenShift cluster:
• 3 control-plane VMs.
• 3 initial worker VMs.
• 0 bootstrap VM after installation.
• Additional workers are created by MachineAutoscaler/ClusterAutoscaler.

Temporary installation state:
• 1 bootstrap VM.
• 3 control-plane VMs.
• 3 workers.
Therefore 8 OpenShift VMs can exist during installation, then the bootstrap VM is removed.

External infrastructure:
• 1 connected mirror host.
• 1 disconnected bastion/installer host.
• 1 mirror registry for a POC; use an HA registry design for production.
• 1 HAProxy VM for the POC; 2 HAProxy VMs plus HA/floating VIPs for production.
• 2 DNS servers recommended.
• 2 NTP sources recommended.
• Existing vCenter/ESXi infrastructure.

Stable OpenShift endpoints:
• api.<cluster>.<base-domain> → API VIP → HAProxy/ALB → masters:6443.
• api.<cluster>.<base-domain> → API VIP → HAProxy/ALB → masters:22623 for Machine Config Server.
• *.apps.<cluster>.<base-domain> → Ingress VIP → HAProxy/ALB → nodes running ingress-router pods.

The API load balancer must be Layer 4/raw TCP/SSL passthrough and stateless. Do not configure session persistence for the Kubernetes API. Red Hat also requires the API health check to use /readyz and requires failed API backends to be removed within the documented tolerance. Machine Config Server traffic on 22623 is internal-only.

# 2. Authoritative references

Use the official Red Hat pages below as the final authority if this runbook and the product documentation ever differ.

OCP 4.20 — Installing on VMware vSphere:
https://docs.redhat.com/en/documentation/openshift_container_platform/4.20/html/installing_on_vmware_vsphere/index

OCP 4.20 — IPI on vSphere:
https://docs.redhat.com/en/documentation/openshift_container_platform/4.20/html/installing_on_vmware_vsphere/installer-provisioned-infrastructure

OCP 4.20 — Disconnected vSphere IPI:
https://docs.redhat.com/en/documentation/openshift_container_platform/4.20/html/installing_on_vmware_vsphere/installer-provisioned-infrastructure#installing-restricted-network-vsphere_installer-provisioned-infrastructure

OCP 4.20 — User-managed load balancer:
https://docs.redhat.com/en/documentation/openshift_container_platform/4.20/html/installing_on_vmware_vsphere/user-provisioned-infrastructure

OCP 4.20 — Disconnected environments / oc-mirror:
https://docs.redhat.com/en/documentation/openshift_container_platform/4.20/html/disconnected_environments/about-disconnected-environments

OCP 4.20 — oc-mirror v2:
https://docs.redhat.com/en/documentation/openshift_container_platform/4.20/html/disconnected_environments/about-installing-oc-mirror-v2

OCP 4.20 — Machine management:
https://docs.redhat.com/en/documentation/openshift_container_platform/4.20/observability/machine_management/index

OCP 4.20 — Cluster autoscaling:
https://docs.redhat.com/en/documentation/openshift_container_platform/4.20/html/machine_management/applying-autoscaling

OCP 4.20 — MachineHealthCheck:
https://docs.redhat.com/en/documentation/openshift_container_platform/4.20/html/machine_management/deploying-machine-health-checks

Red Hat OpenShift Downloads:
https://console.redhat.com/openshift/downloads

Broadcom/VMware documentation:
https://docs.broadcom.com/

HAProxy documentation:
https://docs.haproxy.org/

# 3. Support and version gate — DO THIS FIRST

Before building anything, record the exact versions.

Required:
• OpenShift 4.20.z target release.
• vCenter version/build.
• ESXi version/build.
• vSphere virtual hardware version.
• CPU microarchitecture.
• NSX version, if NSX is present.
• Registry product/version.
• RHEL version for bastion/HAProxy.
• DNS/NTP platform.
• HAProxy version.
• Architecture: normally amd64/x86_64.

OCP 4.20 vSphere requirements include vSphere 8.0 Update 1 or later, or supported VMware Cloud Foundation 5.0+ / VMware vSphere Foundation 9+ configurations. The vSphere hypervisor requires virtual hardware version 15 or later and x86-64-v2 or higher. CSI migration is enabled by default on the supported vSphere releases.

Do not continue until your vSphere release is confirmed against the current Red Hat Tested Integrations/support matrix.

# 4. VM inventory — exact count and responsibility

Recommended POC inventory:

1. CONNECTED-MIRROR01 — 1 VM
   OS: RHEL 9.x
   4–8 vCPU, 16–32 GB RAM, 500 GB+ working disk.
   Internet access.
   Runs oc, openshift-install, oc-mirror v2 and downloads.

2. OCP-BASTION01 — 1 VM
   OS: RHEL 9.x
   4 vCPU, 16 GB RAM, 100 GB+ disk.
   Runs the disconnected installation, DNS/LB checks and oc administration.

3. REGISTRY01 — 1 VM for POC
   Registry: supported Docker V2-2 registry.
   4–8 vCPU, 16–32 GB RAM, storage sized for the complete image set plus growth.
   Production: use an HA registry/storage design rather than one VM.

4. HAPROXY01 — 1 VM for POC
   RHEL 9.x, 2–4 vCPU, 4–8 GB RAM.
   Production: HAPROXY01 + HAPROXY02 with HA/floating VIPs.

5. DNS01/DNS02 — existing or 2 VMs recommended.
6. NTP01/NTP02 — existing or redundant sources.

OpenShift machines provisioned by IPI:
7. Bootstrap — 1 temporary VM, automatically created by installer, deleted after bootstrap.
8. Master0/Master1/Master2 — 3 VMs, automatically created.
9. Worker0/Worker1/Worker2 — 3 VMs initially, automatically created.
10. Worker3..N — automatically created later by MachineAutoscaler/ClusterAutoscaler.

Minimum OpenShift host count is bootstrap + 3 control-plane + at least 2 compute. This runbook deliberately starts with 3 compute machines for HA and autoscaling.

Important: in IPI you do NOT manually create the bootstrap/master/worker VMs. The installer provisions them through vCenter.

# 5. VM specifications

Official minimum OCP 4.20 vSphere values:

Bootstrap: RHCOS, 4 vCPU, 16 GB RAM, 100 GB storage, 300 IOPS.
Control plane: RHCOS, 4 vCPU, 16 GB RAM, 100 GB storage, 300 IOPS.
Compute: RHCOS, 2 vCPU, 8 GB RAM, 100 GB storage, 300 IOPS.

Recommended for this POC:
Bootstrap: 4 vCPU / 16 GB / 120 GB.
Masters: 8 vCPU / 32 GB / 150 GB each.
Workers: 8 vCPU / 32 GB / 150 GB each.
HAProxy: 4 vCPU / 8 GB.
Bastion: 4 vCPU / 16 GB.
Mirror host: 8 vCPU / 32 GB.
Registry: 8 vCPU / 32 GB plus storage sized for images.

These recommended values are engineering choices, not Red Hat minimums. Workloads require additional capacity.

Do not use memory ballooning. Control-plane memory must be committed at or above the required minimum. Compute memory reservations must also meet the published minimum. Use fast storage for etcd; Red Hat calls out a 10 ms p99 fsync target for etcd.

# 6. Example network plan

The following values are examples only.

Base domain: example.com
Cluster name: ocp01
Cluster domain: ocp01.example.com
Machine subnet: 10.20.10.0/24
API VIP: 10.20.10.10
Ingress VIP: 10.20.10.11
Bootstrap: 10.20.10.20
Master0: 10.20.10.21
Master1: 10.20.10.22
Master2: 10.20.10.23
Worker0: 10.20.10.31
Worker1: 10.20.10.32
Worker2: 10.20.10.33
Registry: 10.20.20.10
Bastion: 10.20.20.20
HAProxy01: 10.20.20.21
HAProxy02: 10.20.20.22
DNS01: 10.20.20.53
DNS02: 10.20.20.54

Required DNS:
api.ocp01.example.com -> 10.20.10.10
*.apps.ocp01.example.com -> 10.20.10.11
bootstrap.ocp01.example.com -> 10.20.10.20
master0.ocp01.example.com -> 10.20.10.21
master1.ocp01.example.com -> 10.20.10.22
master2.ocp01.example.com -> 10.20.10.23
worker0.ocp01.example.com -> 10.20.10.31
worker1.ocp01.example.com -> 10.20.10.32
worker2.ocp01.example.com -> 10.20.10.33

Create forward and reverse records according to your DNS policy. The API and wildcard apps records must resolve to the load-balancer frontend addresses.

# 7. Firewall matrix

Allow at minimum:

Bastion -> vCenter TCP 443.
Control-plane/cluster nodes -> vCenter/ESXi TCP 443.
Cluster nodes -> mirror registry TCP 443 (or your registry port).
Clients -> API VIP TCP 6443.
HAProxy -> masters/bootstrap TCP 6443.
Cluster nodes -> API VIP TCP 6443.
HAProxy -> masters/bootstrap TCP 22623.
Cluster nodes -> MCS VIP TCP 22623 as required.
External clients -> Ingress VIP TCP 80/443.
HAProxy -> ingress backend TCP 80/443.
All required hosts -> DNS TCP/UDP 53.
All required hosts -> NTP UDP 123.

Do not expose 22623 broadly. It is the Machine Config Server port and should be restricted to the required internal traffic.

# 8. vCenter preparation

On vCenter, prepare:

• Datacenter.
• vSphere cluster(s).
• Datastore/storage policy.
• Port group/network.
• Folder.
• Tag category.
• Tag.
• vCenter service account.
• Failure domains if using multiple zones.

For HA, distribute the three control-plane VMs across different physical ESXi hosts and, where possible, across independent vSphere failure domains. Multiple failure domains are required to enable region/zone topology.

The installer requires vCenter and ESXi TCP/443 access. Verify that the vCenter service account has the permissions required by the OCP 4.20 vSphere IPI installer; do not guess a permission set. Use the exact Red Hat documented permissions for the release.

# 9. Connected mirror host — install tools

MACHINE: CONNECTED-MIRROR01

Download the exact OCP 4.20.z:
• oc client
• openshift-install
• oc-mirror v2
• Red Hat pull secret

Download page:
https://console.redhat.com/openshift/downloads

Create:

sudo mkdir -p /opt/ocp-downloads
sudo chmod 755 /opt/ocp-downloads
cd /opt/ocp-downloads

# After downloading/extracting the exact release-matched binaries:
sudo install -m 0755 oc /usr/local/bin/oc
sudo install -m 0755 openshift-install /usr/local/bin/openshift-install
sudo install -m 0755 oc-mirror /usr/local/bin/oc-mirror

oc version
openshift-install version
oc mirror version

# 10. Create secure pull-secret storage

MACHINE: CONNECTED-MIRROR01

Obtain the pull secret from the Red Hat Hybrid Cloud Console. Do not paste it into documentation or source control.

sudo mkdir -p /secure/openshift
sudo chmod 700 /secure/openshift

sudo cp pull-secret.json /secure/openshift/pull-secret.json
sudo chmod 600 /secure/openshift/pull-secret.json

# Verify the file exists without printing its contents:
ls -l /secure/openshift/pull-secret.json

# 11. Configure oc-mirror v2

MACHINE: CONNECTED-MIRROR01

First decide exactly which Operators and additional images the cluster needs. Do not mirror every available Operator unless you have a reason; the image set can become very large.

Example ImageSetConfiguration:

The exact channels, versions and Operator packages must be adjusted to the release and Operators you actually require. Validate the file with the current OCP 4.20 oc-mirror v2 documentation.

mkdir -p /opt/ocp-mirror
cd /opt/ocp-mirror

cat > imageset-config.yaml <<'EOF'
apiVersion: mirror.openshift.io/v2alpha1
kind: ImageSetConfiguration
mirror:
  platform:
    architectures:
    - amd64
    channels:
    - name: stable-4.20
      minVersion: 4.20.x
      maxVersion: 4.20.x
  operators:
  - catalog: registry.redhat.io/redhat/redhat-operator-index:v4.20
    packages:
    - name: <operator-name>
      channels:
      - name: <channel>
  additionalImages: []
EOF

# Review:
cat imageset-config.yaml

# 12. Mirror to disk — fully disconnected workflow

MACHINE: CONNECTED-MIRROR01

For a fully disconnected environment, mirror the selected image set to disk, transfer it through the approved security boundary, and then publish it to the target registry.

Run:

export PULL_SECRET=/secure/openshift/pull-secret.json
export MIRROR_DIR=/opt/ocp-mirror

oc mirror --v2   -c $MIRROR_DIR/imageset-config.yaml   file://$MIRROR_DIR

# Inspect generated content:
find $MIRROR_DIR -maxdepth 4 -type f | sort | head -200

# 13. Publish mirror content — disconnected registry

MACHINE: REGISTRY01

Transfer the oc-mirror output/workspace from CONNECTED-MIRROR01 to REGISTRY01 using your approved transfer mechanism.

The registry must support Docker V2-2. The registry must remain available while the cluster is running. For production, make the registry highly available and back it up.

# Example after the transferred mirror workspace is available:
export MIRROR_DIR=/opt/ocp-mirror

oc mirror --v2   -c $MIRROR_DIR/imageset-config.yaml   --from file://$MIRROR_DIR   docker://registry.ocp01.example.com

# Verify registry:
curl -k https://registry.ocp01.example.com/v2/

# 14. Extract the installer from the exact release

MACHINE: CONNECTED-MIRROR01 or OCP-BASTION01, depending on your approved transfer workflow.

The safest approach is to use the installer associated with the exact OCP release being deployed. Preserve the release metadata and installer version together.

# Example if extracting from a release payload:
oc adm release extract   --command=openshift-install   --registry-config=/secure/openshift/pull-secret.json   <MIRRORED_RELEASE_IMAGE>

./openshift-install version

# 15. Disconnected registry trust

MACHINE: OCP-BASTION01

If the mirror registry uses an internal/private CA, obtain the CA certificate from the registry/security team.

Do not use insecure registry mode in production simply to bypass certificate problems. Install the trusted CA according to the OCP 4.20 disconnected installation documentation and your organization's certificate management process.

Test from the bastion:

getent hosts registry.ocp01.example.com
curl -vk https://registry.ocp01.example.com/v2/

# If authentication is required:
podman login registry.ocp01.example.com

# 16. DNS validation gate

MACHINE: OCP-BASTION01

Do not continue until every DNS check succeeds.

dig +short api.ocp01.example.com
dig +short apps.ocp01.example.com
dig +short bootstrap.ocp01.example.com
dig +short master0.ocp01.example.com
dig +short master1.ocp01.example.com
dig +short master2.ocp01.example.com
dig +short worker0.ocp01.example.com
dig +short worker1.ocp01.example.com
dig +short worker2.ocp01.example.com

# Reverse DNS examples:
dig -x 10.20.10.21
dig -x 10.20.10.22
dig -x 10.20.10.23

# 17. NTP validation gate

MACHINE: OCP-BASTION01

Verify NTP from the infrastructure hosts. vCenter, ESXi, DNS, registry, bastion, HAProxy and cluster nodes must use consistent time.

timedatectl
chronyc tracking
chronyc sources -v

# 18. HAProxy installation — POC

MACHINE: HAPROXY01

Install RHEL 9.x using your organization's approved RHEL source. Configure the host's IP, hostname, DNS and NTP first.

Then install HAProxy from your approved RHEL repository.

If SELinux is enforcing and HAProxy must bind to the OpenShift ports, Red Hat's OCP 4.20 example requires the haproxy_connect_any SELinux boolean.

sudo dnf install -y haproxy

sudo setsebool -P haproxy_connect_any 1

sudo systemctl enable haproxy

# 19. HAProxy configuration

MACHINE: HAPROXY01

The following is an example aligned to the OCP 4.20 user-managed LB requirements. Red Hat's documented HAProxy example uses Layer-4 TCP for ingress/MCS and requires API /readyz health checking.

For production, use two HAProxy nodes and a floating frontend VIP. The POC can use one node.

During installation:
• 6443: bootstrap + masters.
• 22623: bootstrap + masters.
• 80/443: workers/ingress nodes.

After bootstrap:
• remove bootstrap from 6443 and 22623.

For a production deployment, implement the /readyz API health monitor and remove an unhealthy API backend within the documented maximum 30 seconds.

sudo cp /etc/haproxy/haproxy.cfg /etc/haproxy/haproxy.cfg.backup

sudo tee /etc/haproxy/haproxy.cfg >/dev/null <<'EOF'
global
    log 127.0.0.1 local2
    pidfile /var/run/haproxy.pid
    maxconn 4000
    daemon

defaults
    mode tcp
    log global
    option dontlognull
    timeout connect 10s
    timeout client 1m
    timeout server 1m
    timeout check 5s

frontend ocp-api
    bind 10.20.10.10:6443
    default_backend ocp-api

backend ocp-api
    balance roundrobin
    option tcp-check
    server bootstrap 10.20.10.20:6443 check
    server master0 10.20.10.21:6443 check
    server master1 10.20.10.22:6443 check
    server master2 10.20.10.23:6443 check

frontend ocp-mcs
    bind 10.20.10.10:22623
    default_backend ocp-mcs

backend ocp-mcs
    balance roundrobin
    option tcp-check
    server bootstrap 10.20.10.20:22623 check
    server master0 10.20.10.21:22623 check
    server master1 10.20.10.22:22623 check
    server master2 10.20.10.23:22623 check

frontend ocp-http
    bind 10.20.10.11:80
    default_backend ocp-http

backend ocp-http
    balance source
    server worker0 10.20.10.31:80 check
    server worker1 10.20.10.32:80 check
    server worker2 10.20.10.33:80 check

frontend ocp-https
    bind 10.20.10.11:443
    default_backend ocp-https

backend ocp-https
    balance source
    server worker0 10.20.10.31:443 check
    server worker1 10.20.10.32:443 check
    server worker2 10.20.10.33:443 check
EOF

sudo haproxy -c -f /etc/haproxy/haproxy.cfg
sudo systemctl restart haproxy
sudo systemctl status haproxy --no-pager

# 20. Test HAProxy before OpenShift installation

MACHINE: OCP-BASTION01

All four endpoints must be reachable before the installer is started.

nc -vz 10.20.10.10 6443
nc -vz 10.20.10.10 22623
nc -vz 10.20.10.11 80
nc -vz 10.20.10.11 443

curl -k https://api.ocp01.example.com:6443/version

# 21. Prepare OCP-BASTION01

MACHINE: OCP-BASTION01

Install the tools you need. Use your organization's RHEL repository configuration.

Store:
• openshift-install
• oc
• pull-secret
• final install-config.yaml
• generated manifests/logs
• kubeconfig

Protect secrets. Never commit them to Git.

sudo dnf install -y jq curl bind-utils nmap-ncat podman

sudo mkdir -p /opt/ocp/install/ocp01
sudo chmod 700 /opt/ocp/install/ocp01

oc version
openshift-install version
oc mirror version

# 22. vSphere failure domains

MACHINE: OCP-BASTION01 / vCenter administrator

For a simple POC you may have one failure domain. For a production-capable topology, use multiple vSphere failure domains if you have real independent compute/storage/network boundaries.

At least one failure domain is required for vSphere IPI. Multiple failure domains enable region/zone topology.

Do not invent zones that all share the same physical ESXi cluster, datastore and network and then claim physical failure isolation. The failure-domain design must correspond to real infrastructure separation.

# 23. Final install-config.yaml

MACHINE: OCP-BASTION01

Create /opt/ocp/install/ocp01/install-config.yaml.

The exact vSphere failure-domain topology must match your vCenter inventory. The example below shows the critical structure. The pull secret and vCenter password are placeholders.

If using the disconnected mirror, include the mirror configuration generated by the OCP 4.20 oc-mirror workflow exactly as documented for the release. Do not manually invent deprecated imageContentSources mappings.

cat > /opt/ocp/install/ocp01/install-config.yaml <<'EOF'
apiVersion: v1

baseDomain: example.com

metadata:
  name: ocp01

sshKey: 'ssh-ed25519 AAAA_REPLACE_ME'

compute:
- name: worker
  replicas: 3
  platform:
    vsphere: {}

controlPlane:
  name: master
  replicas: 3
  platform:
    vsphere: {}

networking:
  networkType: OVNKubernetes

  machineNetwork:
  - cidr: 10.20.10.0/24

  clusterNetwork:
  - cidr: 10.128.0.0/14
    hostPrefix: 23

  serviceNetwork:
  - 172.30.0.0/16

platform:
  vsphere:
    loadBalancer:
      type: UserManaged

    apiVIPs:
    - 10.20.10.10

    ingressVIPs:
    - 10.20.10.11

    vcenters:
    - server: vcenter01.example.com
      user: ocp-installer@example.com
      password: 'REPLACE_ME'
      datacenters:
      - DC01

    failureDomains:
    - name: fd-zone-a
      region: region-a
      zone: zone-a
      server: vcenter01.example.com
      topology:
        datacenter: DC01
        computeCluster: /DC01/host/OCP-Cluster
        datastore: /DC01/datastore/OCP-Datastore
        networks:
        - OCP-Network

pullSecret: 'REPLACE_WITH_PULL_SECRET'
EOF

chmod 600 /opt/ocp/install/ocp01/install-config.yaml
sed -n '1,220p' /opt/ocp/install/ocp01/install-config.yaml

# 24. Validate install configuration

MACHINE: OCP-BASTION01

Before create cluster:
• Verify cluster name/domain.
• Verify API VIP.
• Verify ingress VIP.
• Verify vCenter server.
• Verify datacenter.
• Verify compute cluster.
• Verify datastore.
• Verify network.
• Verify failure domain.
• Verify registry/mirror configuration.
• Verify pull secret.
• Verify SSH key.

Do not paste passwords or pull secrets into support tickets.

cd /opt/ocp/install/ocp01

./openshift-install version

# Optional syntax/manifest preparation:
./openshift-install create manifests   --dir /opt/ocp/install/ocp01

# Review generated manifests if you are making documented customizations.
find /opt/ocp/install/ocp01/manifests -maxdepth 1 -type f | sort

# 25. Start IPI installation

MACHINE: OCP-BASTION01

This is the point where the installer starts provisioning vSphere.

The installer will:
1. Connect to vCenter.
2. Provision the bootstrap machine.
3. Provision control-plane machines.
4. Provision compute machines.
5. Bootstrap Kubernetes/OpenShift.
6. Finish the control plane.
7. Remove/allow cleanup of the bootstrap machine after bootstrap completes.

You should watch vCenter, HAProxy and the installer log at the same time.

cd /opt/ocp/install/ocp01

./openshift-install create cluster   --dir /opt/ocp/install/ocp01   --log-level=info

# 26. Monitor installation

MACHINE: OCP-BASTION01

Use separate terminal sessions.

Terminal A — installer log:

tail -f /opt/ocp/install/ocp01/.openshift_install.log

# 26A. Monitor HAProxy during installation

MACHINE: HAPROXY01

sudo journalctl -u haproxy -f

# 26B. Monitor vSphere

MACHINE: vCenter

Watch for:
• bootstrap VM powered on;
• masters created;
• workers created;
• VM IPs;
• datastore capacity;
• network connectivity;
• vCenter task failures.

If the installer fails, do not immediately delete everything. Capture the installer log, vCenter task error and HAProxy status first.

# 27. Wait for bootstrap completion

MACHINE: OCP-BASTION01

Do not remove bootstrap from HAProxy until this command succeeds.

cd /opt/ocp/install/ocp01

./openshift-install wait-for bootstrap-complete   --dir /opt/ocp/install/ocp01   --log-level=debug

# 28. Remove bootstrap from HAProxy

MACHINE: HAPROXY01

After bootstrap-complete succeeds:
1. Remove bootstrap:6443 from the API pool.
2. Remove bootstrap:22623 from the Machine Config Server pool.
3. Keep masters in both pools.
4. Validate configuration.
5. Reload HAProxy.

Do not remove bootstrap before bootstrap-complete succeeds.

sudo vi /etc/haproxy/haproxy.cfg

# Then validate:
sudo haproxy -c -f /etc/haproxy/haproxy.cfg

sudo systemctl reload haproxy

# Verify ports:
sudo ss -lntp | egrep ':(80|443|6443|22623)\b'

# 29. Complete installation

MACHINE: OCP-BASTION01

cd /opt/ocp/install/ocp01

./openshift-install wait-for install-complete   --dir /opt/ocp/install/ocp01   --log-level=debug

export KUBECONFIG=/opt/ocp/install/ocp01/auth/kubeconfig

oc whoami
oc get clusterversion
oc get nodes -o wide
oc get co
oc get mcp

# 30. First health gate

MACHINE: OCP-BASTION01

Do not proceed to autoscaling/auto-healing until:
• all intended nodes are Ready;
• ClusterVersion is healthy;
• ClusterOperators are Available and not Degraded;
• MachineConfigPools are healthy;
• API and console are reachable;
• registry is reachable;
• storage is functional.

oc get co
oc get nodes -o wide
oc get mcp
oc get clusterversion
oc get machines -n openshift-machine-api
oc get machinesets -n openshift-machine-api

# A quick view of unhealthy pods:
oc get pods -A | egrep -v 'Running|Completed'

# 31. vSphere CSI/storage validation

MACHINE: OCP-BASTION01

Verify CSI and storage classes.

oc get csidrivers
oc get pods -n openshift-cluster-csi-drivers
oc get sc -o wide

cat <<'EOF' | oc apply -f -
apiVersion: v1
kind: PersistentVolumeClaim
metadata:
  name: csi-test
  namespace: default
spec:
  accessModes:
  - ReadWriteOnce
  resources:
    requests:
      storage: 10Gi
EOF

oc get pvc csi-test -w

# After successful test:
oc delete pvc csi-test

# 32. Understand Machine API before autoscaling

MACHINE: OCP-BASTION01

Check that the cluster is using a vSphere platform and Machine API is operational.

oc get infrastructure cluster -o jsonpath='{.status.platform}{"\n"}'
oc get machines -n openshift-machine-api
oc get machinesets -n openshift-machine-api
oc get machineconfigpools

# 33. ClusterAutoscaler

MACHINE: OCP-BASTION01

ClusterAutoscaler decides when cluster capacity should grow or shrink.

It does not itself define which MachineSet to scale. A MachineAutoscaler is required for each compute MachineSet that you want the ClusterAutoscaler to scale.

Start conservatively. The values below are examples and must be adjusted to your vSphere capacity and workload requirements.

cat > /opt/ocp/install/ocp01/cluster-autoscaler.yaml <<'EOF'
apiVersion: autoscaling.openshift.io/v1
kind: ClusterAutoscaler
metadata:
  name: default
spec:
  resourceLimits:
    maxNodesTotal: 20
    cores:
      min: 12
      max: 160
    memory:
      min: 48
      max: 640
  scaleDown:
    enabled: true
    delayAfterAdd: 10m
    delayAfterDelete: 5m
    delayAfterFailure: 30s
    unneededTime: 10m
    utilizationThreshold: "0.5"
EOF

oc apply -f /opt/ocp/install/ocp01/cluster-autoscaler.yaml

oc get clusterautoscaler
oc describe clusterautoscaler default

# 34. MachineAutoscaler

MACHINE: OCP-BASTION01

First find the exact MachineSet names:

oc get machinesets -n openshift-machine-api

# 34A. Create MachineAutoscaler for a worker MachineSet

MACHINE: OCP-BASTION01

Replace <EXACT_MACHINESET_NAME> with the real MachineSet name from the previous command.

If you create multiple worker MachineSets for multiple zones, create a MachineAutoscaler for each MachineSet.

cat > /opt/ocp/install/ocp01/machine-autoscaler-worker.yaml <<'EOF'
apiVersion: autoscaling.openshift.io/v1beta1
kind: MachineAutoscaler
metadata:
  name: worker-autoscaler
  namespace: openshift-machine-api
spec:
  minReplicas: 3
  maxReplicas: 12
  scaleTargetRef:
    apiVersion: machine.openshift.io/v1beta1
    kind: MachineSet
    name: <EXACT_MACHINESET_NAME>
EOF

oc apply -f /opt/ocp/install/ocp01/machine-autoscaler-worker.yaml

oc get machineautoscaler -n openshift-machine-api
oc describe machineautoscaler worker-autoscaler -n openshift-machine-api

# 35. MachineHealthCheck — worker auto-healing

MACHINE: OCP-BASTION01

MachineHealthCheck watches machines managed by a MachineSet or Control Plane MachineSet. When a configured health condition persists for its timeout, the unhealthy Machine can be deleted and a replacement created.

For workers, start with one unhealthy machine allowed at a time.

Important: selectors must match the actual MachineSet labels. The example selector below intentionally uses the generic worker labels; for a multi-zone design, add the exact MachineSet label to scope each health check appropriately.

cat > /opt/ocp/install/ocp01/worker-mhc.yaml <<'EOF'
apiVersion: machine.openshift.io/v1beta1
kind: MachineHealthCheck
metadata:
  name: worker-healthcheck
  namespace: openshift-machine-api
spec:
  selector:
    matchLabels:
      machine.openshift.io/cluster-api-machine-role: worker
      machine.openshift.io/cluster-api-machine-type: worker
  unhealthyConditions:
  - type: Ready
    status: "False"
    timeout: 300s
  - type: Ready
    status: Unknown
    timeout: 300s
  maxUnhealthy: 1
  nodeStartupTimeout: 20m
EOF

oc apply -f /opt/ocp/install/ocp01/worker-mhc.yaml

oc get machinehealthcheck -n openshift-machine-api
oc describe machinehealthcheck worker-healthcheck -n openshift-machine-api

# 36. Control-plane auto-healing

MACHINE: OCP-BASTION01

Treat control-plane auto-healing as a separate, higher-risk exercise.

Before enabling it:
• verify Control Plane Machine Set status;
• verify vSphere provisioning can replace a control-plane VM;
• verify etcd quorum;
• verify storage latency;
• verify only one control-plane machine can be remediated at a time.

If you configure a MachineHealthCheck for the control plane, Red Hat explicitly recommends maxUnhealthy: 1. This protects against automatic remediation when multiple control-plane nodes are unhealthy, which can indicate an etcd problem or an ongoing replacement operation.

Do not power off two control-plane VMs during testing.

oc get controlplanemachineset -n openshift-machine-api
oc get machines -n openshift-machine-api   -l machine.openshift.io/cluster-api-machine-role=master

oc get pods -n openshift-etcd -o wide

# If/when a control-plane MHC is designed and approved:
# maxUnhealthy must be 1 for the control plane.

# 37. Dedicated ingress worker design

For predictable external load balancing, create dedicated ingress worker MachineSets in production.

The OpenShift IngressController should run only on the designated ingress workers. The external LB should target those nodes.

This avoids a failure mode where a router pod moves to a node that is not present in the LB backend pool.

For autoscaling ingress nodes, the LB backend membership must be updated by controlled automation or another supported mechanism when nodes are added/removed.

# 38. Autoscaling acceptance test

MACHINE: OCP-BASTION01

Test in a controlled POC window.

1. Record current machines.
2. Deploy a workload with sufficient resource requests to create pending pods.
3. Watch the scheduler and Machine API.
4. Confirm a new Machine is created.
5. Confirm vCenter creates the VM.
6. Confirm the VM joins as Ready.
7. Confirm the workload schedules.
8. Remove the workload.
9. Wait for scale-down.
10. Confirm the Machine is removed safely.

Commands:

oc get machines -n openshift-machine-api
oc get nodes -o wide
oc get pods -A

# During scale-out:
oc get machines -n openshift-machine-api -w
oc get nodes -w

# After the test:
oc get machines -n openshift-machine-api
oc get nodes

# 39. Auto-healing acceptance test

MACHINE: OCP-BASTION01 + vCenter

Test exactly one worker.

1. Identify a worker.
2. Record the Machine object.
3. In vCenter, perform a controlled failure of that worker.
4. Watch node condition.
5. Wait for MHC timeout.
6. Verify Machine remediation.
7. Verify replacement Machine/VM.
8. Verify replacement Node Ready.
9. Verify workload recovery.

Commands:

oc get nodes -o wide
oc get machines -n openshift-machine-api -o wide

oc get nodes -w
oc get machines -n openshift-machine-api -w

oc describe machinehealthcheck worker-healthcheck   -n openshift-machine-api

# 40. HAProxy failure test

For a production-capable design, do not call a single HAProxy VM highly available.

POC test:
• stop HAProxy and verify expected outage.

Production test:
• HAProxy01 + HAProxy02 with a floating VIP;
• stop active HAProxy;
• VIP must move to standby;
• API and applications must remain reachable.

Validate from an external client:

curl -k https://api.ocp01.example.com:6443/version
curl -k https://console-openshift-console.apps.ocp01.example.com

# 41. HAProxy → NSX ALB migration plan

Keep the OpenShift endpoint identity stable.

Before migration:
• Build NSX ALB in parallel.
• Create API pool: masters:6443.
• Create MCS pool: masters:22623.
• Create ingress pool: ingress workers:80/443.
• Configure Layer-4/TCP passthrough.
• Configure health monitoring.
• Test using temporary VIPs.

Cutover:
1. Freeze LB configuration changes.
2. Verify ALB backends.
3. Transfer ownership of the production API VIP.
4. Transfer ownership of the production Ingress VIP.
5. Test API.
6. Test console.
7. Test application routes.
8. Test worker registration/operations.
9. Keep HAProxy available for rollback until the acceptance window passes.

Do not change api.<cluster>.<domain> or *.apps.<cluster>.<domain> simply because the LB implementation changes.

# 42. Backup and disaster recovery

Autoscaling and auto-healing are not backups.

Back up:
• OpenShift cluster state using the supported backup solution.
• Application persistent data.
• Mirror registry content and registry configuration.
• DNS.
• HAProxy/keepalived configuration.
• Install configuration and generated manifests, with secrets protected.
• MachineAutoscaler/ClusterAutoscaler/MachineHealthCheck manifests.
• vSphere storage policies and relevant inventory configuration.

Perform a restore test. A backup that has never been restored is not a validated DR solution.

# 43. Disconnected upgrade process

For each future OCP update:
1. Confirm supported upgrade path.
2. Mirror the new release.
3. Mirror required Operator content.
4. Verify digest/content.
5. Ensure registry capacity.
6. Test in lower environment.
7. Update the disconnected cluster using the supported update process.
8. Monitor ClusterVersion and ClusterOperators.
9. Retain required old release content according to your rollback/retention policy.

Never delete mirror content just because the cluster has moved to a new release.

# 44. Troubleshooting decision tree

INSTALLER FAILS IMMEDIATELY:
• Check install-config.yaml.
• Check vCenter credentials/permissions.
• Check vCenter/ESXi 443.
• Check DNS.
• Check datastore/network.
• Check release/mirror configuration.

BOOTSTRAP DOES NOT COMPLETE:
• Check HAProxy 6443 and 22623.
• Check bootstrap IP/DNS.
• Check registry access.
• Check vCenter tasks.
• Check time synchronization.
• Check installer log.

NODES DO NOT JOIN:
• Check DNS.
• Check 22623.
• Check registry.
• Check NTP.
• Check MTU/network.
• Check Machine Config Operator logs.

API UNAVAILABLE:
• Check API VIP.
• Check master backends.
• Check /readyz.
• Check etcd.
• Check HAProxy.
• Check firewall.

INGRESS UNAVAILABLE:
• Check IngressController pods.
• Check router node placement.
• Check LB backend list.
• Check 80/443.
• Check wildcard DNS.

AUTOSCALER DOES NOT SCALE:
• ClusterAutoscaler exists?
• MachineAutoscaler exists?
• MachineSet target name correct?
• Pending pods have resource requests?
• vSphere capacity available?
• Machine API healthy?

MHC DOES NOT REMEDIATE:
• Selector matches Machines?
• Machine is MachineSet-managed?
• Condition persisted for timeout?
• maxUnhealthy exceeded?
• nodeStartupTimeout appropriate?

# 45. Final production go-live checklist

Use this checklist as the formal acceptance gate.

INFRASTRUCTURE
[ ] vSphere version supported.
[ ] vCenter supported.
[ ] Virtual hardware >= required version.
[ ] x86-64-v2 or supported architecture.
[ ] ESXi hosts NTP synchronized.
[ ] No memory ballooning.
[ ] Control-plane storage meets performance requirement.
[ ] Failure domains documented.
[ ] vCenter service account least privilege.
[ ] Datastore capacity supports maximum worker count.

NETWORK/DNS
[ ] API VIP reserved.
[ ] Ingress VIP reserved.
[ ] api.<cluster>.<domain> resolves to API VIP.
[ ] *.apps.<cluster>.<domain> resolves to Ingress VIP.
[ ] Reverse DNS completed where required.
[ ] Firewall rules validated.
[ ] 22623 restricted.
[ ] NTP redundant.
[ ] DNS redundant.

REGISTRY/DISCONNECTED
[ ] Mirror registry supports Docker V2-2.
[ ] Mirror registry highly available for production.
[ ] OCP 4.20 release mirrored.
[ ] Required Operators mirrored.
[ ] Registry TLS/CA trusted.
[ ] Registry backup configured.
[ ] oc-mirror workspace retained.
[ ] Disconnected upgrade process documented.

LOAD BALANCING
[ ] API is L4/TCP passthrough.
[ ] API has no session persistence.
[ ] /readyz health check configured.
[ ] API unhealthy backend removed within 30 seconds.
[ ] 22623 configured.
[ ] Ingress 80/443 configured.
[ ] Bootstrap removed after bootstrap completion.
[ ] HAProxy configuration backed up.
[ ] Production HAProxy pair tested.

OPENSHIFT
[ ] 3 control-plane nodes Ready.
[ ] >=3 worker nodes Ready.
[ ] ClusterOperators healthy.
[ ] MachineConfigPools healthy.
[ ] vSphere CSI validated.
[ ] PVC provisioning tested.
[ ] ClusterAutoscaler configured.
[ ] MachineAutoscaler configured for every scalable MachineSet.
[ ] Worker MachineHealthCheck tested.
[ ] Control-plane recovery tested.
[ ] Monitoring/alerting configured.
[ ] Backup/restore tested.
[ ] Upgrade procedure tested.

MIGRATION
[ ] NSX ALB design documented.
[ ] API pool defined.
[ ] MCS pool defined.
[ ] Ingress pool defined.
[ ] Temporary ALB VIP tested.
[ ] HAProxy rollback path retained.

# 46. Quick command sheet

The following is the minimum command set you will repeatedly use from the bastion after installation.

export KUBECONFIG=/opt/ocp/install/ocp01/auth/kubeconfig

# Cluster:
oc whoami
oc get clusterversion
oc get co
oc get nodes -o wide
oc get mcp

# Machine API:
oc get machines -n openshift-machine-api
oc get machinesets -n openshift-machine-api
oc get machineautoscaler -n openshift-machine-api
oc get machinehealthcheck -n openshift-machine-api
oc get clusterautoscaler

# Storage:
oc get sc
oc get pv
oc get pvc -A
oc get csidrivers

# Events:
oc get events -A --sort-by='.lastTimestamp' | tail -100

# Ingress:
oc get ingresscontroller -n openshift-ingress-operator
oc get pods -n openshift-ingress -o wide
oc get routes -A

# Network/DNS:
dig +short api.ocp01.example.com
dig +short apps.ocp01.example.com
curl -k https://api.ocp01.example.com:6443/version

# Machines:
oc get machines -n openshift-machine-api -o wide

# 47. What the installer creates automatically

This distinction is critical for anyone following the guide.

YOU CREATE/PROVIDE:
• vCenter and ESXi infrastructure.
• DNS.
• NTP.
• Network/port group.
• datastore/storage policy.
• vCenter credentials.
• mirror registry.
• bastion.
• mirror host.
• external load balancer.
• API VIP and Ingress VIP.
• install-config.yaml.
• pull secret.
• required firewall rules.

IPI CREATES AUTOMATICALLY:
• temporary bootstrap VM.
• control-plane VMs.
• initial worker VMs.
• associated Machine objects/MachineSets.
• cluster networking components.
• OpenShift services.

AFTER INSTALLATION:
• bootstrap is removed.
• ClusterAutoscaler can request more capacity.
• MachineAutoscaler changes MachineSet replica counts.
• vSphere Machine API creates new worker VMs.
• MachineHealthCheck can remove unhealthy Machines so replacements are created.

The exact generated MachineSet names depend on your cluster name and failure-domain topology; always obtain them with `oc get machinesets -n openshift-machine-api` rather than guessing names.

# 48. Important corrections and safety notes

1. This document uses IPI because that is the requested architecture. Red Hat's OCP 4.20 documentation identifies multiple installation methods and provides a specific restricted-network vSphere IPI procedure using an internal mirror.

2. A true disconnected IPI deployment requires mirrored release content and a registry accessible to the cluster. A proxy does not remove that requirement if the cluster is being treated as disconnected.

3. A single HAProxy VM is acceptable for the POC but is not a production HA load balancer.

4. A single registry VM is acceptable for a POC only. Production requires registry availability appropriate to the OpenShift cluster.

5. The minimum OCP machine sizes are installation minimums, not workload sizing.

6. MachineHealthCheck is powerful but should be introduced gradually. Start with worker remediation and test it. For control-plane MHC, use maxUnhealthy=1.

7. The exact oc-mirror configuration depends on the OCP z-stream and required Operator set. Do not use a generic operator catalog/package list without validating it.

8. The exact vSphere failureDomains section depends on your vCenter inventory. Do not copy the example paths unchanged.

9. Keep API and Ingress VIPs stable if you want to make the future HAProxy-to-NSX-ALB transition low-risk.

10. Before production deployment, validate the exact OCP 4.20.z release, vSphere build, registry, HAProxy and NSX ALB versions against the current Red Hat/Broadcom support matrices.

# 49. Official source notes

This runbook was checked against current OCP 4.20 Red Hat documentation retrieved on 26 August 2026. Key verified points include:
• vSphere supported versions and virtual hardware requirements;
• bootstrap/control-plane/compute machine counts;
• official minimum CPU/RAM/storage/IOPS;
• vSphere IPI disconnected installation requirements;
• user-managed LB ports and behavior;
• oc-mirror v2 and mirror-registry requirement;
• ClusterAutoscaler/MachineAutoscaler relationship;
• MachineHealthCheck behavior and control-plane maxUnhealthy guidance.

Because OCP is a moving product, re-check the exact 4.20.z documentation before implementation, especially Operator catalogs, oc-mirror syntax, vSphere tested integrations and upgrade paths.

# Reference links — clickable

Red Hat OpenShift Downloads: https://console.redhat.com/openshift/downloads

OCP 4.20 Installing on VMware vSphere: https://docs.redhat.com/en/documentation/openshift_container_platform/4.20/html/installing_on_vmware_vsphere/index

OCP 4.20 vSphere IPI: https://docs.redhat.com/en/documentation/openshift_container_platform/4.20/html/installing_on_vmware_vsphere/installer-provisioned-infrastructure

OCP 4.20 Disconnected environments: https://docs.redhat.com/en/documentation/openshift_container_platform/4.20/html/disconnected_environments/about-disconnected-environments

OCP 4.20 oc-mirror v2: https://docs.redhat.com/en/documentation/openshift_container_platform/4.20/html/disconnected_environments/about-installing-oc-mirror-v2

OCP 4.20 Machine management: https://docs.redhat.com/en/documentation/openshift_container_platform/4.20/observability/machine_management/index

OCP 4.20 Cluster autoscaling: https://docs.redhat.com/en/documentation/openshift_container_platform/4.20/html/machine_management/applying-autoscaling

OCP 4.20 MachineHealthCheck: https://docs.redhat.com/en/documentation/openshift_container_platform/4.20/html/machine_management/deploying-machine-health-checks

Broadcom VMware documentation: https://docs.broadcom.com/

HAProxy documentation: https://docs.haproxy.org/